Name :

Mr. Graham

Physics / Period:

April, 2005

Website of interest: www.colorado/edu/physics/phet

- Under the program “Battery-Resistor Circuit”, what is the relationship (direct, inverse, none) between the following?

- resistance and current

- voltage and current

- Under the program “Ohm’s Law”, what is the current through a resistor with the following resistances?  Let voltage = 6 V

a.

R = 200 ohms	i = 		R = 300 ohms 		i =

R= 400 ohms  	i = 		R = 500 ohms		i =

- Do you answers make sense?  Explain!

- What are two ways to increase the resistance in a wire?

a.

b.

Check your answers above under the program “Resistance in a Wire”.

Under the program “Circuit Construction Kit”, answer the following questions.

1.  Construct a circuit with one battery and one light bulb.  Show a sketch below using appropriate symbols.  This is an example of a simple circuit.

- Click on “show values” and calculate the current using Ohm’s Law (R = V / i).

- Place an ammeter in series with the circuit and confirm your answer.

- Place a voltmeter across the battery (in parallel to the battery).  What is the potential increase (voltage rise) across the battery?

- Place a voltmeter across the lightbulb.  What is the voltage drop across the lightbulb?

- What can you conclude about the potential increase across the battery and the voltage drop across the lightbulb?

2.  Construct a circuit with one battery and two light bulbs.  Arrange the light bulbs so that electrons have only one path from which to choose.  Show a sketch below and use appropriate symbols.  This is an example of a series circuit.

- Click on “show values”.

- Place an ammeter in series with the circuit at any two locations.  Why is the ammeter reading the same anywhere in a series circuit?

- Place a voltmeter across the battery.  What is the potential increase across the battery?

- Place a voltmeter across each lightbulb.  What is the voltage drop across each?

- What can you conclude about the voltage drop in a series circuit across each light bulb (of equal resistance) with respect  to the potential increase across the battery?

3.  Construct a circuit with one battery and two light bulbs.  Arrange the light bulbs so that electrons have two paths from which to choose.  Show a sketch below and use appropriate symbols.  This is an example of a parallel circuit.

- Click on “show values”.

- Place an ammeter in series just before the first split in the wire (the junction).  What is the ammeter reading at this location?

- Place an ammeter in series with the circuit just after the split / junction and before each light bulb.  What are the ammeter readings for both locations?

- What can you conclude about the current before and after the junction?

- Place a voltmeter across the battery.  What is the potential increase across the battery?

- Place a voltmeter across each lightbulb.  What is the voltage drop across each?

- What can you conclude about the voltage drop across each light bulb (of equal resistance) in a parallel circuit.