Name, Date___________________________

Parallel Circuits

- Simple Parallel Circuit:  Two Resistors

  - ۞ Clear the screen and set up a simple circuit identical to the setup from part 1.  Make sure the battery is a 10-Volt battery.  Then, connect a second 10-Ω light bulb in parallel to the first bulb.  This means that the circuit splits for the 2 bulbs.

  - **Use the voltmeter to measure the voltages across each bulb.  How do these 2 voltages compare to each other?  How do they compare to the voltage of the battery?

  - **Increase the resistance of one bulb to 100 Ω, then repeat step 4(b).  Did the voltages change significantly?  How?

  - **What can be said, in general, about the voltages for components that are connected in parallel to each other?

  - Change the resistance of the second bulb from 100 Ω to 10 Ω.   Using the ammeter reading, and the known voltage of the battery, use Ohm’s Law to calculate the equivalent resistance of the circuit.

Req = ____________

  - Change one bulb from 10 Ω to 20 Ω, and repeat step 4(e).

Req = ____________

  - Change the 20 Ω bulb to 30 Ω, and repeat 4(e).

Req = ____________

  - ۞ Try to determine the pattern developing.   How is the equivalent resistance related to the values of the resistors used in a parallel circuit?

  - ۞ Using the formula given to you by your instructor, predict the equivalent resistance of 3 10-Ω bulbs that are connected in a parallel circuit.

  - After showing your prediction to your instructor, confirm your prediction by setting up a 3-bulb circuit on the computer and using the same technique from 4(e).

- Practice Problem

  - Parallel Circuits

    - Sketch a parallel circuit that contains the following:  a 9.0-V battery, 3 15-Ω light bulbs, and a 40-Ω light bulb in parallel.

    - Calculate Req for this circuit.  What is the voltage across each bulb?

    - Calculate the current flowing through one of the bulbs.

    - Explain why automobile headlights must be wired in parallel rather than series.